# Operating the coin quad v3 box

Double count mode (position 9 on top rotary switch)

Set bottom switch back to 0 count down normal mode

Coin Quad Board Operation

J2 – Short across J2 for delay mode, gives 2 second delay

If the mode sw (bottom rotary switch) is in position 1 or 7 delay is variable 0 -3 seconds.

When you first short across J2 it will display total count

So to see total count select delay momentarily

Bottom selector switch is for selection of options (BCD switch 4)

Position

0 normal operation

1 Normal operation but delay is random if self pickle mode (delay) is selected

2 display zero

3 display zero

4 display other BCD switches (2&3) in the display

5 display zero

6 count up mode, adds count for each shot

7 count up mode, adds count for each shot if delay selected give variable delay

8 zeros again

9 infrared mode

To reset coin count to zero turn bottom selector switch to 8 and back to zero, or

Programming port

Pin 5 is 5 volts

Pin 1 is the RB3 input which is normally pulled down

When they are shorted through a switch, the shot count is reset.

Top switch is for extra counts when coin added of zero, this applies to position 0 to 8

If there is 000 in display,

Then when coin added, top switch number is also added to the coin count input.

For position 9 the double count is enabled and there is no zero add count function.