# Testing the coin box

Power up box

Check voltage input to regulator, should be about 9 volts

Check output from regulator, should be 5.0 volts

Set the bottom BCD Switch to 3, so the display shows 888

Click pickle inputs to be sure switches operate a relays properly

Count should stay at 888

Check zero count coin add (top switch setting value)

Check each pickle for add or subtract shot count in display

Check power off, power on, save shot count

Check Double count mode (position 9 on top rotary switch)

Set bottom switch back to 0

Set top 3 switches to 1

Press coin switch 9 times and watch the digits

See that 1 through 9 works on each display digit

Power off to return display to zero

Set all three switches to 2

Press coin switch and be sure the display adds 222 to the count

Repeat for numbers 3 through 9

Be sure all the BCD switches work in all positions

Check pickle switch, holding it to see it clicks about every half second or so

Check three switch inputs from pickle and extra switch, one, two then both

Be sure you get the proper count down on the display

Coin Quad Board Operation

J2 – Short across J2 for delay mode, gives 2 second delay

When you first short across J2 it will display total count

Bottom selector switch is for selection of options (BCD switch 4)

Position

0 normal operation

1 Normal operation but delay is random if self pickle mode selected

2 display zero

3 display zero

4 display other BCD switches (2&3) in the display

5 display zero

6 count up mode, adds count for each shot

7 count up mode, adds count for each shot if delay selected give variable delay

8 zeros again

9 infrared mode

To reset coin count to zero turn bottom selector switch to 8 and back to zero.

Be sure when there are counts in display and you zero using the position 9 that you can power off and back on and it stays zero.

Top switch is for extra counts when coin added of zero, this applies to position 0 to 8

If there is 000 in display,

Then when coin added, top switch number is also added to the coin count input.

For position 9 the double count is enabled and there is no zero add count function.

Switch input on programming port is for zeroing the count,

Need to verify this

Pin 5 is 5 volts

Pin 1 is the RB3 input which is normally pulled down

When they are shorted through a switch, the shot count is reset.