PROGRAMING THE  ELFIPA RECIECER TO THE TRANSMITTER

On the receiver of the wireless control you will see a red button, this is the learn button.  Here is how you teach the receiver to respond to a manual remote transmitter (not voice release) to release a target by pressing button #1;

Manual remote controllers

Receiver

……….press AND hold the red learn button. DO NOT RELEASE THE RED BUTTON.

Transmitter

……. Press and release button #1 on the transmitter.

Now you can release the red learn button on receiver……………. that’s it, you’re done…..

Another example …..You want to teach it to release on buttons numbers 1,2,3,and 4. Do the following;

Receiver

……….press AND hold the red learn button. DO NOT RELEASE THE RED BUTTON.

Transmitter

……. Press and release button #1  (do not release red button on receiver), Press and release button #2  (do not release red button on receiver), , Press and release button #3  (do not release red button on receiver), and Press and release button #4 on the transmitter…….

Now you can release the red learn button on receiver……………. that’s it, you’re done…..

VOICE RELEASE PROGRAMMING…. Sporting clay version

To program a sporting clay voice release stand it is almost the same process. Here is how you would teach it to release target on the number 1 button.

Receiver

……….press AND hold the red learn button. DO NOT RELEASE THE RED BUTTON.

Transmitter

……. Press and release button #1, now loudly say the word “PULL” (you have just transmitted the signal)

Now you can release the red learn button on receiver……………. that’s it, you’re done…..

You can also teach this sporting voice release to throw on multiple buttons…let say you want that machine to release on buttons # 1,2,3,4………do  this……

Receiver

……….press AND hold the red learn button. DO NOT RELEASE THE RED BUTTON.

Transmitter

……. Press and release button #1, now loudly say the word “PULL” (do not release red button on receiver), Press and release button #2, now loudly say the word “PULL” (do not release red button on receiver), Press and release button #3, now loudly say the word “PULL” (do not release red button on receiver), Press and release button #4, now loudly say the word “PULL”.

Now you can release the red learn button on receiver……………. that’s it, you’re done…..the trap will release if you press 1 on voice release  stand and then call  “pull” or the trap will release if you press 2 on voice release stand and then call  “pull”……same with buttons 3 and 4

VOICE RELEASE PROGRAMMING…. Skeet version

The same works for skeet voice release stand. Here is how you teach it for skeet.

Receiver- Low House

……….press AND hold the red learn button in the low house. DO NOT RELEASE THE RED BUTTON.

Transmitter

Low house… Press and release button L , now loudly say the word “PULL” (do not release red button on receiver), Press and release button #D, now loudly say the word “PULL” …..Now you can release the red learn button on receiver

Receiver-High House

……….press AND hold the red learn button in the low house. DO NOT RELEASE THE RED BUTTON.

Transmitter

Low house… Press and release button H, now loudly say the word “PULL” (do not release red button on receiver), Press and release button #D, now loudly say the word “PULL” …..Now you can release the red learn button on receiver               …that’s it….your done

For ATA trap it is the same thing…..just decide what button you want the machine to release on, teach it and you’re good to go!