WOBBLE BOX INSTRUCTIONS

1. Locate trap to the desired location.

2. Hook up 10 ten connector from control box to wobble control receptor.

3. Hook up control box to power source.

4. By the end of the launching arm you will find a “ Arm Lockout” Pin (hitch pin),

Remove It!

5. You will notice the control box has five switches. See below;

Top left               Power ON/ OFF

Top Right           Disable ON/OFF  (this function allows you to disable the release

system when loading trap just in case, inadvertently,

somebody would release a target at a dangerous

time).

Bottom left       Release   ( Allows for a manual release of the target at the control box)

Bottom Right…. Horizontal        (Horizontal control of trap)

Manual      Allows you to manually control  the horizontal

movement of trap, which permits you to set it at a

desired location for fixed (non oscillating) shooting.

Auto           Normal setting for wobble trap.  The trap will oscillate

randomly to a new horizontal position after each shoot.

Bottom Center ….  Vertical          (Vertical control of trap)

Manual      Allows you to manually control  the vertical

movement of trap, which permits you to set it at a

desired location for fixed (non oscillating) shooting.

Auto           Normal setting for wobble trap.  The trap will oscillate

randomly to a new vertical position after each shoot.

Other Functions;

ATA TRAP SHOOTING….set vertical height to desired locations and turn vertical switch to middle position (OFF).  Leave Horizontal switch on Auto.

FIXED TARGET shooting ( no oscillation)  set both vertical and horizontal switch to middle position (off) after setting the trap to the desired height and angle by using the vertical and horizontal manual switch.

WIDER ANGELS.  	You will note on the base of the trap there is a vented access door. To change to wider or narrow horizontal angles remove knob #531 and reinsert it in the hole next two it.